SCORE:________		Name:__________________________________________

ECE 4100 Advanced Computer Architecture

Midterm Exam

1. (15 points) A hardware enhancement to a computer system runs 9 times faster when it is used.  The speedup that results when using the enhancement was 3.5.  What percentage of the original program code was able to use the new hardware enhancement?

Percentage of code that uses new hardware enhancement  _____________

2. (10 points) What Program characteristic normally makes a 1-bit Branch Predictor a poor choice and why does a 2-bit Predictor typically perform better?

3. (5 points) What Program characteristic is exploited to increase the branch prediction accuracy of correlating and tournament branch predictors over basic 2-bit branch prediction hardware?

4. (10 points) What advantages does Tomasulo’s algorithm have over a scoreboard?

5. (10 points) A single-issue processor has a pipeline CPI of 1.7 and a cache miss rate of 3% per instruction and a 2Ghz clock. Another company’s version of the processor has a pipeline CPI of 1.1 and a clock rate of 1.5Ghz, but with a cache miss rate of 4%. The main memory access time is 20ns for a miss. Compute the MIPS performance of the two processors including both pipeline and cache CPI.

First Processor execution rate 	= __________ MIPS

Second Processor execution rate 	= __________ MIPS

6. (5 points) What problem does a re-order buffer solve in a dynamically scheduled processor?

7. (20 points) Identify all of the dependences in the code segment shown below (1 loop iteration only) and place your answers in the table below:

Inst. Number

LOOP: 	L.D 		F4, 0(R1)			1

L.D 		F3, 0(R2)			2

MULT 	F5, F3, F4			3

SUB.D 	F4, F5, F10			4

ADD.D	F8, F8, F4			5

## DIV.D  	F4, F3, F5			6

DADDIU 	R1, R1, #-8			7

BNE  		R1, R3 LOOP		8

| Type of Dependency (RAW, WAW, or WAR) | Instruction Pair (i,j) (e.g., 1,4) |
| --- | --- |

8. (25 points) Consider the program segment below running on a single-issue machine using Tomasulo’s Algorithm. Fill in the clock cycle number in the table below assuming the latencies shown in the table below the program. Last, be sure to answer the question below the table.

L.D 		F4, 0(R1)

L.D 		F3, 0(R2)

MULT 	F5, F3, F4

SUB.D 	F4, F5, F10

ADD.D 	F8, F8, F4

### DIV.D  	F4, F3, F5

DADDIU 	R1, R1, #-8

BNE  		R1, R3 LOOP

Details of Functional Units:

| Unit | Latency (in Execute) | Reservation Stations |
| --- | --- | --- |
| FP Add/Sub | 4 | 2 |
| FP Mult | 6 | 2 |
| FP Div | 10 | 2 |
| FP Load/Store | 2 | 2 each Load/Store buffers |
| Integer Unit | 1 | None |

Note: The FP arithmetic units are NOT pipelined – i.e. you must wait for the current operation to finish execution before using the unit again. You can do a new FP load/store every clock cycle. The WB stage takes only 1 clock cycle and during that cycle the new data appears on the CDB and at all reservation stations.

| Instruction | Issue | Execute | WB |
| --- | --- | --- | --- |
| L.D F4, 0(R1) | 0 |  |  |
| L.D F3, 0(R2) |  |  |  |
| MULT F5, F3, F4 |  |  |  |
| SUB.D F4, F5, F10 |  |  |  |
| ADD.D F8, F8, F4 |  |  |  |
| DIV.D F4, F3, F5 |  |  |  |
| DADDIU R1, R1, #-8 |  |  |  |
| BNE R1, R3 LOOP |  |  |  |

If a reservation station was added to the integer unit would any of the numbers above change?