EE 4077 Xilinx Computer Interface Project Ideas from  GT ECE Student Projects

Voice Recorder with Playback, CDROM player

need microphone and speaker with A/D and D/A chip

Telephone answering machine or Voice mail system

need Xicom telephone interface chip and DTMF chip perhaps and A/D and D/A chip

Caller ID System

Chip Tester or PROM Programmer

Digital Scope, Digital Voltmeter, Spectrum Analyzer

need high speed and high accuracy A/D chip and perhaps SRAMs to store data

Function Generator

need function generator chip and D/A chip

Home Security System

need door and windows contacts and perhaps passive IR

Weather Station

need weather instruments to interface to computer and A/D chip

Universal Remote Control Device

need remote IR chips

CCD Camera Interface

need CCD Camera

Cable Descrambler, Digitally Tuned Radio

need TV or VCR for video lab demo and special purpose video or radio chips

Digital Thermostat

need thermocouple and A/D chip - also used a fan to cool and a light to heat with a 7seg 	temperature display

Radio Control Interface

need Radio Control hardware and servos to interface

Bio Med Type Interface  (Audio Tester and EKG)

need Bio med sensors, op amps and A/D hardware

Logic Analyzer (general purpose or specific to PC bus)

use SRAMS to store data

Robot Control

need servos and robot model - have seen cars, a crane, and an arm.

can build PWM or PPM into Xilinx chip to control servos directly

RS232 Com Port

Use Xilinx to build a UART and a MAX232 chip for TTL to EIA conversion

Sonar

Need Polariod Sonar Kit or perhaps one of the Sonar Tape Measure Devices

Parts available from lab TA

Xilinx X4003-5 84 pin PLCC package, misc TTL chips, 741 OP Amps, LEDs,

High Speed 16K by 4 Static RAMS, 7SEG LEDs, LCD display panels from NCR

The easy to use National A/D and D/A chips are in short supply in the lab

Don’t forget to buffer any Xilinx outputs used to drive external devices!

For other parts check Digi Key and Jameco catalogs in the lab

Digi Key sells Xilinx chips - Jameco sells the Xilinx PLCC to DIP adapter boards used in the lab

Local stores such as ACK Radio,  Radio Shack or other distributors

also can be used.  Only the more common parts are stocked locally.