Score: _________________    Name:_______________________________________________

EE 4077 Test II

Test is open book and notes.

Place all answers in the spaces provided.

1. (5%) A design,  “proj”,  has xmake errors which file(s) should you check for details?

2.(5%)  How could you use a logic probe to see if  DRQ4 was not being used?

3.(5%)  Give three reasons that explain why local bus slots can offer higher bandwidth bus cycles.

4.(5%)  A slow CMOS logic chip with internal data latches is attached to an 8-bit output port on an 8Mhz system.  It has a very long setup time that is 75ns. too long to work using a standard bus cycle.  How could you fix it using the minimum amount of hardware and what hardware is required (i.e. just a summary of the hardware is needed not a full logic diagram).

5.(5%)  What are the logic 0 and 1 levels on an RS232 serial port.

6.(5%)  Why would you expect a  P6 chip to be only around 50% faster than a Pentium it they both had  the same clock frequency?

7.(15%)  Fill in the timing diagram below for an interrupt cycle.  The hardware is set up as shown in fig. 6-8 on page 150 of the text and uses the PC AT bus signal, IRQ5.  Interrupts are enabled for IRQ5 and a interrupt request occurs.  The interrupt is serviced and it continues to repeat on a periodic basis.  Use X if there are any don't care signals.  The correct sequence of events is all that is important in the diagram since the actual times will vary.  Assume INTR and INTA are active high signals.

Timing Diagram for Interrupt Operation

Int. Request  ____------_______________________________________________________________---

from device

Int. Routine______________________----------------------------------_____________________________

Executing

IRQ5

INTR

INTA

Port bit X

Port bit Y

The CPU will find the starting address of the interrupt routine at ___________________(hex address)

8.(20%) Design an 1-bit output port on a Xilinx chip using parts from the Xilinx library.  At power on reset the output bit should go to logic 0.  A handshake line sent from the external device, GOT_IT, signals that data has been loaded from the port indicating that another byte can be output to the port.  A handshake line form the computer, TAKE_IT, signals that new data is available for the external device.  GOT_IT must reset TAKE_IT in the I/O hardware.   TAKE_IT is set whenever the CPU writes to the data port.  The external device has D flip-flops which can operate at bus speeds on the data lines and the TAKE_IT signal clocks the external device’s flip-flops.  The port can use programmed I/O, or interrupt driven I/O.  A control/status port with the next sequential address is used to  disable/enable interrupt requests.  Writing to the data port clears the interrupt request.  Bit 0 on the control port is used to enable/disable interrupt requests.  Bit 0 on the status port reads the TAKE_IT bit.  The starting address of the port is hardwired to 3EC.  The port uses one I/O address for data and the next address for status/control.   Draw the detailed schematic as it would appear in the viewlogic CAD system.  Place labels on all common signal names and clearly label the address decoder section, the I/O port data hardware, I/O port control hardware, and the interrupt request logic.

Viewlogic Schematic  for 1-bit Xilinx I/O output port with interrupt capability

9.(20%) Write two C functions that use the hardware described in the previous problem.  The first C function, OUTBYTE, uses programmed I/O only and waits for the port to be ready (i.e. external device has last data value and is ready for another)and outputs a new data value, DATA, to the port.  DATA is passed as an argument in the OUTBYTE call (i.e. OUTBYTE(DATA) ).  The second program uses interrupts to output the new data value, DATA, to the port (include interrupt service routine C code only - assume interrupts are already enabled and vectors have been setup by main program.)

C function for Programmed I/O

C Interrupt Routine for Interrupt Driven I/O

10. (15%)  Using an 8255 chip with the minimum number of other gates and flip flops, design the hardware required to set up parallel port A for 8-bit DMA driven input operation.  Show the common pin names of any 8255 connections.  The external device clocks the data into port A using a handshake line and this should produce an DRQ4.  Draw a logic diagram in the space below and indicate the mode setup you are using on the 8255.  Hint: Use port C handshake lines