DE2 notes on porting older UP2 and UP3 designs

The following notes will help you understand how the book’s designs were ported to the DE2 and explain the differences you may encounter in the various devices on the DE2 versus information in the book on the other Altera boards.

Clock

Most UP3 designs use the 48Mhz clock. The onboard clock is 50Mhz on the DE2. A DE2 version of the video PLL is provided that makes this correction.

Pushbuttons

DE2 Pushbuttons have a simple hardware debounce circuit built in. In most cases you will not need to debounce them with FPGA hardware. The debounce tutorial in Chapter 4 will always count up by 1, since the pushbuttons are already debounced. The middle two of the four DE2 pushbuttons are used on most of the book’s designs.

LCD display

The LCD controller is compatible with the UP3. An extra output bit must be added and set high to turn on the LCD display (LCD_ON). There is also an LCD backlight pin that can be used (LCD_BLON).

VGA Output

The VGA controller chip needs a video blanking signal (video_blank_out) , and a video clock (video_clock_out). These new outputs must be added. A new version of the VGA sync core for the DE2 outputs these values.

VGA Color Outputs

The video DAC chip on the DE2 supports 8-bit color RGB values or 16 million colors. You can get brighter colors if you wire all 8-bits of each red, green, and blue color input together and connect it to the one color bit output from an older 8 color design. If you just connect to the highest DAC bit, it will work but it will produce colors that are not quite as bright.

The changes below are needed whenever you port an existing design to the DE2. (These changes have already been made in the book’s designs.)

Change device family and device type on any of your designs

Change to the correct Cyclone II device type for the DE2. Device number is EP2C35F672C6.

DE2 Pin assignments

Change all pin assignments to the correct pins for the DE2. See Users manual or DE2 pin table file. Pin assignments are stored in a project’s *.QSF file.