## Downloading Your Design to the DE2 Board

Hooking Up the DE2 Board to the Computer

If you have a UP 2 or UP 3 board refer to the download Sections on those boards. To try your design on a DE2 board shown as seen in Figure 1.13, plug the USB download cable into the DE2 board’s USB connector (leftmost of the three USB connectors on the top left side of the board) and attach the other end to an open USB port on the PC. Using the 9V AC to DC wall transformer attach power to the DC power connector located on the upper left corner of the DE2 board. Press in the power switch located on the upper left edge of the board below the power connector. When properly powered, the blue power LED on the DE2 board next to the power switch should light up and the other LEDs will flash, if it is still setup to run the default demo design shipped from the factory.

Figure 1.13  ALTERA DE2 board showing Pushbutton and LED locations used in design.

Preparing for Downloading

Make sure that you have assigned the correct Device Name for the DE2. The DE2 contains a EP2C35F672C6 Cyclone II FGPA. When you change the device type you will also need to redo the pin assignments. Make sure that you have also assigned the correct pin numbers for the DE2 board. PB1 is pin N23, PB2 is pin P23 and the LED is pin AE23. (refer to section 1.1 for help) Whenever you change the Device or pin assignments, if is necessary to recompile before downloading.

After checking to make sure that the cables and jumpers are hooked up properly, you are ready to download the compiled circuit to the DE2 board. Select Tools  Programmer. Click on Hardware Setup, select the proper hardware, a USB-Blaster. (If a window comes up that displays, "No Hardware" to the right of the Hardware Setup button, use the Hardware Setup button to change currently selected hardware from "No Hardware" to "USB-Blaster". If a red JTAG error message appears or the start button is not working, close down the Programmer window and reopen it. If this still doesn’t correct the problem, then there is something else wrong with the setup or cable connection. Go back to the beginning of this section and check each step and connection carefully.)

Final Steps to Download

The filename orgate.sof should be displayed in the programmer window. The *.sof file contains the FPGA’s configuration (programming) data for your design. To the right of the filename in the Program/Configure column, check the Program/Configure box. To start downloading your design to the board, click on the Start button. Just a few seconds are required to download. If download is successful, a green info message displays in the system window notifying you the programming was successful.

Testing Your Design

On the DE2 board, the middle two pushbuttons are used in the design and one of the LEDs to the left just above them. The locations of PB1, PB2, and the decimal LED are in the lower right corner of the board as seen in Figure 1.13. After downloading your program to the DE2 board, the LED in the lower right corner should turn off whenever a pushbutton is hit. Since the output of the OR gate is driving the LED signal, it should be on when no pushbuttons are hit. Since the buttons are active low, and the BNOR2 gate also has active low inputs and output, hitting either button should turn off the LED.

Congratulations! You have just entered, compiled, simulated, downloaded a design to a FPGA device, and verified its operation. Since you are using a DE2 board, you can skip the sections on the UP 2 or UP 3 board and go directly to Section 1.6.