# Appendix C: UP 1 and UP 2 Pin Assignments

Table C.1 UP 1 and UP 2 board EPM7128S MAX CHIP I/O pin assignments.

| Pin Name | Pin Type | Pin | Function of Pin |
| --- | --- | --- | --- |
| MSD_dp | OUTPUT PIN | 68 | Most Significant Digit of Seven-segment Display Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| MSD_g | OUTPUT PIN | 67 | MSD Display Segment G (0 = LED ON, 1 = LED OFF) |
| MSD_f | OUTPUT PIN | 65 | MSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| MSD_e | OUTPUT PIN | 64 | MSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| MSD_d | OUTPUT PIN | 63 | MSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| MSD_c | OUTPUT PIN | 61 | MSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| MSD_b | OUTPUT PIN | 60 | MSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| MSD_a | OUTPUT PIN | 58 | MSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| LSD_dp | OUTPUT PIN | 79 | Least Significant Digit of Seven-segment Display Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| LSD_g | OUTPUT PIN | 77 | LSD Display Segment G (0 = LED ON, 1 = LED OFF) |
| LSD_f | OUTPUT PIN | 75 | LSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| LSD_e | OUTPUT PIN | 76 | LSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| LSD_d | OUTPUT PIN | 74 | LSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| LSD_c | OUTPUT PIN | 73 | LSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| LSD_b | OUTPUT PIN | 70 | LSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| LSD_a | OUTPUT PIN | 69 | LSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| PB1 | INPUT PIN | * | Pushbutton 1 (non–debounced, 0 = button depressed) |
| PB2 | INPUT PIN | * | Pushbutton 2 (non–debounced, 0 = button depressed) |
| D1..D16 LEDs | OUTPUT PIN | * | 16 Discrete LEDs - D1…D16 (0 = LED ON, 1 = LED OFF) |
| SW1 & SW2 | INPUT PIN | * | MAX DIP Switch Inputs - SWxS1...SWxS8 (1 = Open, 0 = Closed) |
| Clock | INPUT PIN | 83 | 25.175Mhz System Clock on low skew Global Clock Line |
| Prototyping Header Pins | INPUT, OUTPUT | 1-84 | Black Prototyping Headers next to MAX chip – Numbers are silk-screened on board. Pins 12, 33, 54, 75, and 83 are not available. |
| * Jumper wires from the switch or LED to the MAX prototyping headers are required to use these devices. Any available unused MAX header pin can be assigned to this device. |  |  |  |

Table C.2  UP 1 and UP 1X Board  FLEX CHIP I/O pin assignments.

| Pin Name | Pin Type | Pin | Function of Pin |
| --- | --- | --- | --- |
| MSD_dp | OUTPUT PIN | 14 | Most Significant Digit of Seven-segment Display - Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| MSD_g | OUTPUT PIN | 13 | MSD Display Segment G (0 = LED ON,1 = LED OFF) |
| MSD_f | OUTPUT PIN | 12 | MSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| MSD_e | OUTPUT PIN | 11 | MSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| MSD_d | OUTPUT PIN | 9 | MSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| MSD_c | OUTPUT PIN | 8 | MSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| MSD_b | OUTPUT PIN | 7 | MSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| MSD_a | OUTPUT PIN | 6 | MSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| LSD_dp | OUTPUT PIN | 25 | Least Significant Digit of Seven-segment Display - Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| LSD_g | OUTPUT PIN | 24 | LSD Display Segment G (0 = LED ON, 1 = LED OFF) |
| LSD_f | OUTPUT PIN | 23 | LSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| LSD_e | OUTPUT PIN | 21 | LSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| LSD_d | OUTPUT PIN | 20 | LSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| LSD_c | OUTPUT PIN | 19 | LSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| LSD_b | OUTPUT PIN | 18 | LSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| LSD_a | OUTPUT PIN | 17 | LSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| FLEX_switch_1 | INPUT PIN | 41 | FLEX DIP Switch Input 1 (1 = Open, 0 = Closed) |
| FLEX_switch_2 | INPUT PIN | 40 | FLEX DIP Switch Input 2 (1 = Open, 0 = Closed) |
| FLEX_switch_3 | INPUT PIN | 39 | FLEX DIP Switch Input 3 (1 = Open, 0 = Closed) |
| FLEX_switch_4 | INPUT PIN | 38 | FLEX DIP Switch Input 4 (1 = Open, 0 = Closed) |
| FLEX_switch_5 | INPUT PIN | 36 | FLEX DIP Switch Input 5 (1 = Open, 0 = Closed) |
| FLEX_switch_6 | INPUT PIN | 35 | FLEX DIP Switch Input 6 (1 = Open, 0 = Closed) |
| FLEX_switch_7 | INPUT PIN | 34 | FLEX DIP Switch Input 7 (1 = Open, 0 = Closed) |
| FLEX_switch_8 | INPUT PIN | 33 | FLEX DIP Switch Input 8 (1 = Open, 0 = Closed) |
| PB1 | INPUT PIN | 28 | Pushbutton 1 (non–debounced, 0 = button depressed) |
| PB2 | INPUT PIN | 29 | Pushbutton 2 (non–debounced, 0 = button depressed) |
| Horiz_Sync | OUTPUT PIN | 240 | VGA Video Signal - Horizontal Synchronization |
| Vert_Sync | OUTPUT PIN | 239 | VGA Video Signal - Vertical Synchronization |
| Blue | OUTPUT PIN | 238 | VGA Video Signal - Blue Video Data |
| Green | OUTPUT PIN | 237 | VGA Video Signal - Green Video Data |
| Red | OUTPUT PIN | 236 | VGA Video Signal - Red Video Data |
| PS2_CLK | BIDIRECTIONAL | 30 | Clock line for PS/2 Mouse and Keyboard |
| PS2_DATA | BIDIRECTIONAL | 31 | Data line for PS/2 Mouse and Keyboard |
| Clock | INPUT PIN | 91 | 25.175 MHz System Clock on low skew Global Clock Line |

Table C.3  FLEX Expansion B Header Pins.

| Header Pin | FLEX Pin | Header Pin | FLEX Pin |
| --- | --- | --- | --- |
| 1 | Unregulated Power(7.2V) | 2 | GND |
| 3 | VCC (+5V) | 4 | GND |
| 5 | VCC (Right Servo VCC) | 6 | GND (Right Servo VCC) |
| 7 | No connection | 8 | DI1/99 |
| 9 | DI2/92 | 10 | DI3/210 |
| 11 | DI4/212 | 12 | DEV_CLR/209 |
| 13 | DEV_OE/213 | 14 | DEV_CLK2/211 |
| 15 | 109 | 16 | 110 (Right Servo Signal) |
| 17 | 111 | 18 | 113 |
| 19 | 114 | 20 | 115 |
| 21 | 116 | 22 | 117 |
| 23 | 118 | 24 | 119 |
| 25 | 120 | 26 | 126 |
| 27 | 127 | 28 | 128 |
| 29 | 129 | 30 | 131 |
| 31 | 132 | 32 | 133 |
| 33 | 134 | 34 | 136 |
| 35 | 137 | 36 | 138 |
| 37 | 139 | 38 | 141 |
| 39 | 142 | 40 | 143 |
| 41 | 144 | 42 | 146 |
| 43 | 147 | 44 | 148 |
| 45 | 149 | 46 | 151 |
| 47 | 152 | 48 | 153(IR Sensor) |
| 49 | 154 (IR Right LED) | 50 | 156(Compass West) |
| 51 | 157 (IR Left LED) | 52 | 158(Compass East) |
| 53 | 159 | 54 | 161(Compass South) |
| 55 | 162 (Left Servo Signal) | 56 | 163(Compass North) |
| 57 | VCC (Left Servo VCC) | 58 | GND (Left Servo GND) |
| 59 | VCC (+5V) | 60 | GND |
| Note: Pins in parenthesis are used for UP1-bot Interfacing |  |  |  |