Chapter 2

The Altera UP 1 and UP 2 CPLD Boards

# The Altera UP 1 and UP 2 CPLD Boards

The Altera University Program 1 (UP 1), CPLD design laboratory board is shown in Figures 2.1 and 2.2. This board supports both a MAX EPM7128S 2,500 gate, P-term based CPLD device, located on the left side, and a FLEX EPF10K20 20,000 gate, SRAM based CPLD device, located on the right side.

Figures 2.1 and 2.2  The Altera UP 1 and UP 2 board.

The newer UP 2 and UP 1X boards are identical to the UP 1 except that they contain a larger FLEX 10K70 70,000 gate CPLD in place of the FLEX 10K20.

Either or both CPLD devices can be programmed using a JTAG ByteBlaster* cable attached to a PC printer port. Jumpers on the board select which device is programmed.

An external 7-9V DC power supply or a low-cost AC to DC wall adapter can be used to provide power. An on-board 25.175MHz clock oscillator is provided for use by the two CPLD devices and it is prewired to the MAX and FLEX chips at pins 83 and 91 respectively.

## Programming Jumpers

A Byteblaster* programming adapter cable is supplied with the UP 1 board and it connects the UP 1 board to the PC’s parallel port (LPT) for device programming. The printer port mode of the PC should be set in the PC’s BIOS to ECP or EPP.

To select one of the CPLD devices to program, jumpers on the UP 1 board are set to select the MAX or the FLEX CPLD as shown in Table 2.1.

Table 2.1  UP 1 device selection jumpers for programming.

## MAX 7000 Device and UP 1 I/O Features

The MAX EMP7128S 7000 device provides up to 2500 usable gates. It is on the left side of the UP 1 board and is in a socket. It connects to a prototyping header connector surrounding the chip. The UP 1’s 25.175Mhz onboard clock is connected to a global clock buffer pin at pin 83. The MAX digit display has two seven-segment LED displays that are pre-wired to the device. Two eight-position DIP switches, sixteen discreet LEDs, and two SPST momentary contact non-debounced pushbuttons can also be connected to the MAX prototyping header with small wire jumpers. In addition, circuit-board holes are provided for an additional 60-pin expansion header that can be added to connect external hardware. Table 2.3 lists the MAX pin assignments for devices that are already connected to the MAX device.

## MAX and FLEX Seven-segment LED Displays

The UP 1 board contains two dual seven-segment displays. One set is pre-wired to the MAX chip and the other is pre-wired to the FLEX chip. As seen in Figure 2.3, each display contains seven segments and one decimal point for a total of eight LEDs. Each LED is connected to and controlled by output pins on the MAX or FLEX chip. A low on the output pin turns on each LED.

The common connection of all on the display segments is already connected to 5 volts. This is a common practice that makes the LEDs brighter since TTL outputs traditionally provide more current at the low level.

By turning on various segments it is possible to display decimal and hexadecimal numbers in the display. For hexadecimal digits B and D, a lower case "b" and "d" is normally used to avoid confusion with "8" and "0".

Logic is needed to convert from 4-bit binary values to seven-segment values before displaying. The logic used to provide this output is called a seven-segment decoder and is provided in the UP1core libraries as dec_7seg. This circuit is used in Chapter 4.

This logic can also be designed at the gate level by putting the truth table in seven four-variable Karnaugh maps and minimizing the logic. As an alternative, it can be implemented by selecting a seven-segment decoder from the TTL library parts, or automatically minimized and synthesized using a  language like VHDL, Verilog, or AHDL.

Figure 2.3  MAX and FLEX seven-segment LED display segment names.

The truth table for a decimal to seven-segment decoder is shown in Table 2.2.

Table 2.2 Truth table for seven-segment display.

| Display | Bits 3-2-1-0 | Seg a | Seg b | Seg c | Seg d | Seg e | Seg f | Seg g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0000 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | 0001 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 2 | 0010 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 3 | 0011 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 4 | 0100 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 5 | 0101 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 6 | 0110 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 7 | 0111 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 8 | 1000 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 9 | 1001 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |

Table 2.3 UP 1-board EPM7128S MAX CHIP I/O pin assignments.

| Pin Name | Pin Type | Pin | Function of Pin |
| --- | --- | --- | --- |
| MSD_dp | OUTPUT PIN | 68 | Most Significant Digit of Seven-segment Display Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| MSD_g | OUTPUT PIN | 67 | MSD Display Segment G (0 = LED ON, 1 = LED OFF) |
| MSD_f | OUTPUT PIN | 65 | MSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| MSD_e | OUTPUT PIN | 64 | MSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| MSD_d | OUTPUT PIN | 63 | MSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| MSD_c | OUTPUT PIN | 61 | MSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| MSD_b | OUTPUT PIN | 60 | MSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| MSD_a | OUTPUT PIN | 58 | MSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| LSD_dp | OUTPUT PIN | 79 | Least Significant Digit of Seven-segment Display Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| LSD_g | OUTPUT PIN | 77 | LSD Display Segment G (0 = LED ON, 1 = LED OFF) |
| LSD_f | OUTPUT PIN | 75 | LSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| LSD_e | OUTPUT PIN | 76 | LSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| LSD_d | OUTPUT PIN | 74 | LSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| LSD_c | OUTPUT PIN | 73 | LSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| LSD_b | OUTPUT PIN | 70 | LSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| LSD_a | OUTPUT PIN | 69 | LSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| PB1 | INPUT PIN | * | Push-Button 1 (non–debounced, 0 = button depressed) |
| PB2 | INPUT PIN | * | Push-Button 2 (non–debounced, 0 = button depressed) |
| D1..D16 LEDs | OUTPUT PIN | * | 16 Discrete LEDs - D1…D16 (0 = LED ON, 1 = LED OFF) |
| SW1 & SW2 | INPUT PIN | * | MAX DIP Switch Inputs - SWxS1...SWxS8 (1 = Open, 0 = Closed) |
| Clock | INPUT PIN | 83 | 25.175Mhz System Clock on low skew Global Clock Line |
| Prototyping Header Pins | INPUT, OUTPUT | 1-84 | Black Prototyping Headers next to MAX chip Numbers are silk-screened on board. Pins 12, 33, 54, 75, and 83 are not available. |
| * Jumper wires from the switch or LED to the MAX prototyping headers are required to use these devices. Any available unused MAX header pin can be assigned to this device. Note: A number of other pins are pre-assigned and required for programming, power, and ground connections. |  |  |  |

When connecting external hardware, additional pins are available for use on the MAX expansion connector on the board. Refer to the UP 1 University Program Design Laboratory Packages User Guide for details. This manual is supplied with the UP 1 board and is available free along with a manual supplement at http://www.altera.com/.

## FLEX 10K Device and UP 1 I/O Features

The FLEX EPF10K20RC240 device provides up to 20,000 usable gates and is located on the right side of the UP 1 board. With six embedded array blocks (EABs), it can also provide up to 12K bits of memory. The larger FLEX EPF10K70 on the UP 1X board provides up to 70,000 usable gates and it contains nine EABs.

The UP 1’s 25.175Mhz onboard clock is connected to a global clock buffer input at pin 91. This frequency has been selected to provide the exact frequency required to generate 640 by 480 VGA video. The clock can also be divided down using internal logic, when a slower clock frequency is needed.

The FLEX device is attached to a VGA video connector, a PS/2 mouse and keyboard port, two seven-segment LED displays, an eight-position DIP switch, and two SPST momentary contact non-debounced pushbuttons. The VGA and PS/2 connectors have no additional circuitry and provide simple pin connections to the FLEX device. To generate video output, mouse, or keyboard input, an interface must be designed using logic inside the FLEX device. The UP1core library functions that support VGA output along with keyboard and mouse inputs are described in Chapter 5. The internal design of these interface circuits is described in Chapters 9, 10, and 11.

In addition, circuit-board holes are provided for three 60-pin FLEX expansion headers that can be added to connect external hardware to the FLEX chip. One of these expansion headers is used for the robot described in Chapter 12.

The FLEX 10K20 is a surface mount chip and it is soldered directly to the board. It is difficult if not impossible to replace the FLEX chip, so extreme care should be exercised when interfacing FLEX I/O pins to any external devices. Table 2.4 and Appendix C lists the FLEX pin assignments for I/O devices that are already connected to the FLEX device.

When connecting external hardware, additional pins are available for use on the FLEX expansion a,b, and c connectors on the board. Refer to the UP 1, University Program Design Laboratory Packages User Guide for details. This manual comes with the UP 1 or UP 1X board and is available free along with a manual supplement at http://www.altera.com.

Table 2.4  UP 1 Board 10K20RC240 FLEX CHIP I/O pin assignments.

| Pin Name | Pin Type | Pin | Function of Pin |
| --- | --- | --- | --- |
| MSD_dp | OUTPUT PIN | 14 | Most Significant Digit of Seven-segment Display - Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| MSD_g | OUTPUT PIN | 13 | MSD Display Segment G (0 = LED ON, 1 = LED OFF) |
| MSD_f | OUTPUT PIN | 12 | MSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| MSD_e | OUTPUT PIN | 11 | MSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| MSD_d | OUTPUT PIN | 9 | MSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| MSD_c | OUTPUT PIN | 8 | MSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| MSD_b | OUTPUT PIN | 7 | MSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| MSD_a | OUTPUT PIN | 6 | MSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| LSD_dp | OUTPUT PIN | 25 | Least Significant Digit of Seven-segment Display - Decimal Point Segment (0 = LED ON, 1 = LED OFF) |
| LSD_g | OUTPUT PIN | 24 | LSD Display Segment G (0 = LED ON, 1 = LED OFF) |
| LSD_f | OUTPUT PIN | 23 | LSD Display Segment F (0 = LED ON, 1 = LED OFF) |
| LSD_e | OUTPUT PIN | 21 | LSD Display Segment E (0 = LED ON, 1 = LED OFF) |
| LSD_d | OUTPUT PIN | 20 | LSD Display Segment D (0 = LED ON, 1 = LED OFF) |
| LSD_c | OUTPUT PIN | 19 | LSD Display Segment C (0 = LED ON, 1 = LED OFF) |
| LSD_b | OUTPUT PIN | 18 | LSD Display Segment B (0 = LED ON, 1 = LED OFF) |
| LSD_a | OUTPUT PIN | 17 | LSD Display Segment A (0 = LED ON, 1 = LED OFF) |
| FLEX_switch_1 | INPUT PIN | 41 | FLEX DIP Switch Input 1 (1 = Open, 0 = Closed) |
| FLEX_switch_2 | INPUT PIN | 40 | FLEX DIP Switch Input 2 (1 = Open, 0 = Closed) |
| FLEX_switch_3 | INPUT PIN | 39 | FLEX DIP Switch Input 3 (1 = Open, 0 = Closed) |
| FLEX_switch_4 | INPUT PIN | 38 | FLEX DIP Switch Input 4 (1 = Open, 0 = Closed) |
| FLEX_switch_5 | INPUT PIN | 36 | FLEX DIP Switch Input 5 (1 = Open, 0 = Closed) |
| FLEX_switch_6 | INPUT PIN | 35 | FLEX DIP Switch Input 6 (1 = Open, 0 = Closed) |
| FLEX_switch_7 | INPUT PIN | 34 | FLEX DIP Switch Input 7 (1 = Open, 0 = Closed) |
| FLEX_switch_8 | INPUT PIN | 33 | FLEX DIP Switch Input 8 (1 = Open, 0 = Closed) |
| PB1 | INPUT PIN | 28 | Push-Button 1 (non–debounced, 0 = button depressed) |
| PB2 | INPUT PIN | 29 | Push-Button 2 (non–debounced, 0 = button depressed) |
| Horiz_Sync | OUTPUT PIN | 240 | VGA Video Signal - Horizontal Synchronization |
| Vert_Sync | OUTPUT PIN | 239 | VGA Video Signal - Vertical Synchronization |
| Blue | OUTPUT PIN | 238 | VGA Video Signal - Blue Video Data |
| Green | OUTPUT PIN | 237 | VGA Video Signal - Green Video Data |
| Red | OUTPUT PIN | 236 | VGA Video Signal - Red Video Data |
| PS2_CLK | BIDIRECTIONAL | 30 | Clock line for PS/2 Mouse and Keyboard |
| PS2_DATA | BIDIRECTIONAL | 31 | Data line for PS/2 Mouse and Keyboard |
| Clock | INPUT PIN | 91 | 25.175 MHz System Clock on low skew Global Clock Line |

## Obtaining a UP 2 Board and Cables

UP 2 boards are available for purchase from Altera’s University Program at special student pricing. UP 2 board kits now come with a power supply and a heat sink is no longer needed. If you have the older UP 1 or UP 1X boards, you will need to obtain a power supply and heat sink elsewhere:

Power Supply

Jameco (http://www.jameco.com) product number, 127503 - DC910F5, is an AC to DC Wall Transformer that supplies 9VDC at 1000MA and has the proper power connector for the UP 1 and UP 1X boards.

Radio Shack also has some universal AC to DC Wall Transformers that will work, but be careful to get the correct voltage, current, polarity, and a female 2.5MM x 5.5 mm power connector that fits the UP 1 and UP 1X boards.

Supplies over 9V DC and up to 12V DC can be used but they tend to overheat the UP 1's 5V DC on board regulator. For proper voltage regulation, the UP 1’s regulator requires a minimum input of 7 volts. If you plan to use a keyboard or mouse attached to the UP 1 or UP 1X board, a 1000 ma output is recommended, since the UP 1 or UP 1X must also provide power to them.

If you wish to use another power supply other than the one listed above and need a power connector to plug into the board, a coaxial DC power plug 5.5mm O.D. and 2.5mm ID is needed. For example, Radio Shack Number 274-1568C or equivalent will work. Be sure to double check the polarity when wiring up the connector. The outer conductor is ground and the inner conductor is +Vcc.

Heat Sink

The regulator chip gets very hot when large designs using several LEDs are running on the UP 1 or UP 1X. A small heat sink for the board’s voltage regulator is highly recommended. DigiKey (http://www.digikey.com) part number, HS213, can be bolted to the regulator and is small enough to fit in the tight space so that the regulator can be bent back down on the board's surface.

Longer Cable

For any of the boards, a longer 25pin to 25pin PC M/F parallel printer cable is also useful since the Byteblaster* cable provided with the boards is often too short to reach the printer port. All 25 wires must be connected in the printer extension cable. Any computer store should have these cables. A three-foot well-shielded cable works best. Avoid using extra long cables.

PLCC Extraction Tool

Do not attempt to remove the MAX chip from the socket without a PLCC extraction tool. PLCC extraction tools are available from Digikey. The AMP 84 pin PLCC extraction tool works well for this purpose. This tool is only needed for replacing an inoperative MAX CPLD. One PLCC extraction tool per laboratory is generally sufficient.