Laboratory

12

Robot Challenge: A Team Design Project

This lab is designed to give the student experience working in a team environment to complete an extended design project. The rest of the term will be given to complete this project, and a formal group report is required. Teams of four are allowed for this project.

## Overview

Using the simple computer as the UP1-bot controller (instead of a state machine), design a robot that follows the random commands from an standard TV remote control while storing the path that it travels in memory. When the Power button on the IR remote control is pressed, the robot should read the dipswitches to determine the return path. This path will be one of four modifications to the original path:

- Turn 180 degrees and follow the same path back to start (robot going forward). This requires the robot to perform the path in reverse order.

- Do not turn around but backup over the exact path taken.

(This option is presently implemented as the only option when On/Off is pressed.)

- Turn 180 degrees and perform the recorded path. (This is the only pattern that reads the memory path in the order it was recorded.)

- Do not turn around but backup taking a mirrored path.

This lab will discuss several basic components with which you can work (although you are free to design your own components for better efficiency, speed, etc.) and leave the final implementation up to you. There will be a bonus for students who get the robot to receive the dipswitch directive over the IR communications from the TA’s robot instead of from the dipswitches.

## Project Files

To download the latest lab 12 files from \\POMPEII\:

- Run the batch file P:\ASSEMBLE.BAT. This will download all the files discussed in this lab to the directory c:\appl\max2work\tim\.

To download them from the web page, go to Downloads on the digital web page.

## Provided Project Files and Source Code

The provided project files and source code (up1src.src) implement the IR follow and backup steps on the robot. Compile and download this project to a robot. Study how the assembler code works and play with it. Note that the Reset may not reset the indirect address pointer at the beginning so the robot may need to be re-programmed after each run. (You may want to add this.)

Modify the robot but keep the stack path recording and the table for backing up correctly. Study how the backup protocol works and add tables to do the other protocols. (You will likely have to draw several pictures to get this right.) Table 1 is se up to give you a start. If you follow the subsequent examples, you will see how the translation is made.

As an example, notice “K2: Fwd” under the Forward column. This is Key 2 on the remote that generally means “Forward.” On the data stack, a 2 will be recorded indicating that on that part of the path, the robot was moving forward. If “Backup” has been selected as the dipswitch option, during replay the robot should be backing up when a 2 is encountered on the data stack. This means that the 2 now needs to be interpreted the same way that Key 8 was originally interpreted. A table that makes this translation automatically is built once and any random path will be properly retraced while moving backwards. This concept works for the other paths, but the student must build the tables.

Table 1. Translation table with one of the four translation patterns entered.

| Number Keypad for Remote Translation Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Forward | 180 + Fwd Retrace Path | Backup | 180 + Fwd Original | Backup Mirrored |
| K1: Left-Fwd |  | K7: Back-Left |  |  |
| K2: Fwd |  | K8: Backup |  |  |
| K3: Right-Fwd |  | K9: Back-Right |  |  |
| K4: Hard Left |  | K6: Hard Right |  |  |
| K5: Stop |  | K5: Stop |  |  |
| K6: Hard Right |  | K4: Hard Left |  |  |
| K7: Back-Left |  | K1: Left-Fwd |  |  |
| K8: Backup |  | K2: Fwd |  |  |
| K9: Back-Right |  | K3: Right-Fwd |  |  |

The robot is placed on the ground and the reset button is pressed. The TA will drive the robot with the TV remote in a random pattern. This will continue until the TA presses the On/Off button. At this point, the student’s computer reads the dipswitches to determine what the next path will be.

1

4

2

5 : then On/Off

6

3

2

2

2 : Start

2

A sample pattern is illustrated in Figure 1. The 2 (forward button) is pressed to start the robot moving. If a 1 (forward-left) is pressed, the robot will continue to move until another button is pressed. Here when the robot reaches about 45 degrees, the 2 (forward button) is pressed and the robot continues. The 3 (forward-right) is pressed then a 2. The 6 (hard right) is pressed. Again, if nothing else is pressed the robot will just go around in circles. This is why the timer must be read and the time recorded for each key press. Continuing, we interrupt the 6 with a 2 to go forward, then a 4 (hard left), a 2, then finally a 5 (stop). Then On/Off is pressed.

Figure 1. An example pattern as driven by the TA with corresponding remote key entries.

8

9

6

8

5 at Rewind

4

8

8

7

8

5 : Stop

The backup pattern looks like Figure 2. But buttons are NOT being read from the remote – the robot is simply following the stack memory in reverse order while applying a reinterpretation of the buttons. (BTW, the stack grows from zero up, not 255 down as previously indicated. The start of the path will always be stored at zero. The end will be the last location recorded.) Note also that the numbers on the return path diagram are the interpretation of the stack numbers. The recorded times are used exactly.

Figure 2. A properly implemented backup pattern with the reinterpreted keys.

For the other patterns, directions are given to illustrate consistency, however, it is left as an exercise for the student to develop the proper translation tables.

Figure 3. Turn-around and retrace, Turn-around original, and Backup-mirrored patterns with original paths illustrated with a dotted line.

The “forward” patterns (first two patterns in Figure 3.) both involve the robot turning around 180 degrees before it does anything. (It is suggested that this be done with timing and not the compass.) The first pattern is the retrace of the path of the original pattern, but with the robot moving in a forward direction.

The second pattern is the original pattern followed in the order that it was stored. It is not retracing the original path as illustrated by the dotted line.

The last pattern, the backup-mirrored return, is another pattern where the robot does not turn around, but backs up and returns in a mirroring fashion.

You should review the files on the download and study how the original program works. A complete understanding of this process is one of the primary goals of this exercise. Once you understand what has been done, the patterns are relatively simple to implement.

Here are some important files that are included in the download:

- TOP_ROBOT.GDF: The top-level schematic for the example robot.

- SCOMP.VHD: A sample dual-memory simple computer implementation including IN/OUT instructions and Load Indirect (LOADI) and Store Indirect (STORI) for indirect addressing.

- RECEIVER.VHD: (For Bonus) The receiver module will need to be added to top_robot.gdf if you plan to do the bonus problem. Do this only after the other part is working.

- PROGRAM.MIF: The instruction MIF file for this project.

- DATA.MIF: The data MIF file for this project.

- UP1DEF.SRC: The definition file for the simple computer (for use in WinTim).

UP1SRC.SRC: The assembly code file that generates the program.mif file for this project. This is the instruction file you should modify and then regenerate program.mif using WinTim.

## Project Issues

- The computer robot Remote Control sensor implements the following “actions” for the keys on the remote:

| Key | Action |
| --- | --- |
| 1 | Left-hand forward turn |
| 2 | Forward |
| 3 | Right-hand forward turn |
| 4 | Rotate counter-clockwise |
| 5 | Stop |
| 6 | Rotate clockwise |
| 7 | Left-hand backward turn |
| 8 | Backward |
| 9 | Right-hand backward turn |
| On/Off | Turn 180 degrees, and transfer the stored path to a second robot that will then follow the stored path in reverse to return to the starting point. –10% if it just backs up. |

- A timer is used as an external device that can be read and reset to zero. It is accessed with an IN and OUT instruction.

- You may modify the computer to create any new instructions that you need. A “Lookup” instruction macro has been created to simplify getting the value in a table. It does not save any space over typing in the several lines of code each time, it just makes it easier to read. Also, an INCrement Macro has been created to simplify incrementing a location.

- Use WinTim to convert your assembly code to an instruction MIF file.

- As a last step, make the robot designs as small as possible.  Part of your grade will depend on how small you can make it.  Start by deleting unnecessary instructions in the simple computer and rewriting any inefficient VHDL code used in the simple computer.  You can also eliminate modules like the 7-segment displays, which are only used for debugging. From the .rpt file, find out and report the percentage usage.

## WinTim Meta Assembler

### Assembling the Source Code to Run the Code Handout

- Run “WinTim32.EXE” (one of the files you just downloaded).

- Select FILE => OPEN => UP1SRC.SRC (If Label Table appears, click OK)

- Select FILE => OPEN => UP1DEF.SRC

- Click on the UP1DEF.SRC window then select ASSEMBLY => META-ASSEMBLY. (There should be no errors.)

- Click on the UP1SRC.SRC window then select ASSEMBLY => ASSEMBLE USING DEFINITION FILE (There should be no errors.)

- Select OUTPUT => ASSEMBLED DATA (as .MIF). This is a finished .mif file!!!

- Select FILE => SAVE AS and name the file PROGRAM.MIF.

### Compiling your Results (You already know how to do this.)

- Start MAX+plus II and OPEN the TOP_ROBOT.GDF.

- In MAX+plus II set the project to the current file.

- Do a Save and Compile

- Download the .sof to the UP1 board and test.

Your robot should follow commands sent to it from the remote. When On/Off is pressed, it should back up and retrace its path.

You will want to modify the UP1SRC.SRC file to do the first check-off.

The file, UP1SRC.SRC, has been optimized for this particular lab.

When you look at the assembler program notice the first few lines of the Program Area:

ORG		H#00		; Begin program at 00

Start:	LOAD		Zero		; Load 0 into AC

STORE		Stack		; Stack is initialized at 0000

The ORG says that the LOAD instruction is to be stored at 00 and subsequent instructions are to continue from there. The LOAD Zero takes the place of having to write “02 F8” in a .MIF file. It does the same thing. Note that the word Zero has been EQUivalenced on a line at the top of the program. The EQUs are like search-and-replaces in a word processor. They just make it easier to read LOAD Zero instead of LOAD H#F8. But where, you might ask is the H#F8? Look in the file DATA.SRC and the following line will be found.

F8		:	0000;	% ZERO Vector %

The “LOAD Zero” is translated as “LOAD H#F8” which loads the value at F8 which is zero. WinTIM converts everything for you into a standard .MIF file.

### For those who create their own opcodes

The file, UP1DEF.SRC, is a definition file that equates the words ADD with 00, STORE with 01, etc. IMPORTANT: If you use your own computer or a different set of opcode definitions than used in SCOMP, you will need to change this file. It is relatively easy to change. Use the existing file as a template for any changes that you make. If you use the download files, you will not need to change this file.

## TRANSMITTER Module (Only for the Bonus problem)

The TRANSMITTER module is always transmitting a value. On the rising edge of the send signal, the INPUT[15..0] bus is latched and transmission of this value begins. Transmission of this value continues until the next rising edge of the send signal.  Also, note that you should not latch a new value until the old value has completed transmission (waiting about 0.1 seconds between transmissions should be adequate).

The TRANSMITTER module should be connected as follows:

- CLOCK_1KHZ: Connect to a 1KHz clock signal (from the CLK_DIV module).

- SEND: On the rising edge of this signal, the value on INPUT[15..0] is latched and transmitting the new value begins.

- INPUT[15..0]:  This is the value to be transmitted on the next rising edge of send.

- SERIAL_OUT: The signal sent to the infrared diodes on the proximity detector (pins 154 & 157).

## RECEIVER Module (Only for the Bonus problem)

The RECEIVER module is waits for an infrared signal as long as the CLEAR signal is LOW. It receives the serially transmitted bytes and presents 2-byte words in parallel on the DIGIT[15..0] bus.

The RECEIVER module should be connected as follows:

- CLOCK_100KHZ: Connect to a 100KHz clock signal from CLK_DIV.

- CLEAR: When HIGH the receiver state machine is reset.  Must be LOW to receive data.

- SENSOR: The signal from the proximity detector’s infrared sensor (pin 153).

- DIGIT[15..0]:  This is the last received value.  It is reset to 0x0000 when the CLEAR signal is HIGH.

## IR_REMOTE Module

The IR_REMOTE module (available on Pompeii) should be connected as follows:

- CLOCK_100KHZ: Connect to a 100KHz clock signal (from the CLK_DIV module).

- PB: This is for reset.  Connect to a pushbutton or other input.

- IR_INPUT:  This is the input from the IR sensor.  Connect to an input pin assigned to pin 111 (or pin 153 to use the front-facing proximity sensor).

- COMMAND_OUT[7..0]: This is the parallel output from the IR sensor.  This output vector is unique for the number keys and a few others (see next section).

## RCA Remote Control

The RCA remote control should be programmed to TV code 063.  To ensure it is properly programmed, complete the following steps (from the RCA Universal Remote manual):

- Press and hold the CODE SEARCH button until the Indicator Light lights, then release the CODE SEARCH button.

- Press and release the TV button (the Indicator Light will blink then remain lit).

- Enter the code 0 6 3.

When programmed with the TV code 063, the following values (8 bits) will be output from the IR_REMOTE module for each key.

| Key | Code (Hex) | Key | Code (Hex) |
| --- | --- | --- | --- |
| 0 | 59 | Prev Ch | 5A |
| 1 | 45 | Mute | 5D |
| 2 | 55 | Chan  | 51 |
| 3 | 4D | Chan  | 49 |
| 4 | 43 | TV/VCR | 0F |
| 5 | 53 | Enter | -- |
| 6 | 46 | Rew | -- |
| 7 | 47 | Play | -- |
| 8 | 57 | FF | -- |
| 9 | 4F | Rec | -- |
| OFF/ON | 56 | Pause | -- |
| Vol  | 4C | Stop | -- |
| Vol  | 5C |  |  |

## Laboratory Report

- Lab report 12 is a FORMAL REPORT. One report per group is required. Each group member will get the same grade on the report.

- Each group member is required to complete a peer survey for everyone else in his/her group. This peer survey is NOT to be done in the presence of the other members and is not to be shown or discussed with other members.

- Be sure to include properly labeled hard copies of the following:

- Top-level schematics of your UP1-bots.

- Schematics and/or VHDL code for every component you modified or created for this project. Note: In a formal report it is best to include relative samples of the VHDL code in figures as you discuss them and then include the entire VHDL code as an appendix. When properly done, this adds to the flow of the report.

- MIF files used (each line of the program should be documented).